Portable Gaming Console with ESP32-WROVER, TFT Display, and MicroSD Storage

Santiago Galeano Castaño 							Date: 07/23/2025

Summary

ArcadESP is an open-source, portable, retro video game console. Its main utility is to emulate games from classic consoles (such as the NES) using an ESP32 WROVER IE microcontroller, which is a very cheap and powerful chip. The main goal of the project is to leave it as a development board, which allows programming it for more functions.

3D View Reference:

ArcadESP: The Open-Source Retro Gaming Handheld

Description:

The circuit is centered on an ESP32-WROVER-I microcontroller. The main power supply for the system is 3.3V.

The design integrates several key peripherals for operation. It includes a TFT screen for graphics display, managed through multiple GPIO connections of the microcontroller. For user input, the system features a series of buttons, including direction controls (up, down, left, right), action buttons (Start, Select, A, B), and menu buttons, which connect to a PCF8574 I/O port expander.

Data storage is handled through a MicroSD card slot with push-out functionality, allowing the reading and writing of information. Additionally, the circuit incorporates an auto-reset section for the microcontroller. Test points (TP1, TP2, TP3, TP4) are also included for monitoring key voltages such as V_USB, V_BATT and 3.3V.

Resources:

| USB-B | Power and data connector. |
| --- | --- |
| LTC4054 | Integrated circuit for battery charging. |
| MIC5219 3.3v Regulator | Regulates and stabilizes the voltage to 3.3V. |
| ESP32 WROVER-I | The main microcontroller (the brain). |
| CH340 | USB to serial converter for programming. |
| BUTTONS | Buttons for user interaction. |
| PCF8574 I/O Expander | Expands input/output pins for the buttons. |
| TFT Display | Screen for visual output. |
| Micro SD Card | Module for external storage. |
| Audio DAC UDA1334ATS | Converts digital audio to analog. |
| Amplifier PAM8403 | Amplifies the audio signal for output. |
| Jack 3.5mm | Output connector for headphones or speakers. |
| LED Blue | Visual status indicator light. |

Features:

Processing Core: ESP32-WROVER-IE N8R8 module with a dual-core CPU, Wi-Fi, Bluetooth, and integrated PSRAM.

Connectivity & Power: USB-B port for power and programming via an onboard CH340 USB-to-UART controller.

Power Management: Includes an LTC4054 Li-Ion battery charger and an MIC5219 LDO regulator for a stable 3.3V supply.

Audio Subsystem: Features an I2S Stereo DAC (UDA1334ATS), a PAM8403 Class-D amplifier, and a 3.5mm audio jack output.

User Interface: TFT color display for visual output, accompanied by user input buttons and a status LED.

External Storage: Micro SD card slot for expandable data and media storage.

I/O Expansion: PCF8574 I/O expander to provide additional digital pins over the I2C bus.

Applications:

- Retro Emulation Console (Main Application)

- Portable Media Player

- IoT Device and Control Tool

- Development and Learning Platform (All in One)

Power Tree:

▶ Power Source (USB / Battery)

└───▶ Power Systems & Protections

│

├───▶ Charging Circuit (e.g., TP4056)

│      │

│      ├───> Charges the Li-Po Battery (3.7V)

│      │

│      └───> Status LEDs (Charging / Full)

│

└───▶ 3.3V Voltage Regulator

│

└───▶ Main Power Rail (3.3V)

│

├─▶ B. Processing & Storage

│   ├─ Microcontroller (ESP32)

│   └─ Storage (MicroSD Slot)

│

├─▶ C. Output (Display & Audio)

│   ├─ Display (TFT)

│   └─ Audio Output (DAC / Amp)

│

├─▶ D. Inputs & Controls

│   ├─ I/O Expander (I2C)

│   └─ Auto-Reset Circuit

└─▶ E. Debug & Programming

└─ USB to UART Interface

Input Source: Power originates from either a USB connection or a Li-Po Battery.

Charging & Protection: This input power feeds a charging circuit, which manages battery charging and provides a regulated output. This stage also powers the status LEDs.

Main Regulation: Power from the battery or charging circuit is routed to the 3.3V Voltage Regulator. This is the core of the power system, converting the variable battery voltage (typically 3.0V-4.2V) into a stable, constant 3.3V.

Distribution: The 3.3V rail distributes power to all active modules in the device: the microcontroller, display, audio circuitry, SD card slot, I/O expander, and the programming interface.

Power Budget:

The system can be powered by a USB connection or a Li-Po battery. The input power goes through a protection and charging circuit. A central 3.3V voltage regulator then takes this power and supplies a stable 3.3v to all the main components. These components include the ESP32 microcontroller, the TFT display, the audio output, the MicroSD slot, and the input/control circuits.

Typical Use: The device draws 423 mA (2.12 W) from the 5V USB source for normal operation.

Maximum Use: Under heavy load (like using Wi-Fi), the draw spikes to 1.11 A (5.55 W) from the 5V USB source.

Block Diagram:

Schematic:

3D Model Without Components:

The PCB is similar, but there are differences since the original idea is that it can be used as a development board.

Temperature:

| Component | Category | Operating Temperature Range | Additional Notes |
| --- | --- | --- | --- |
| LTC4054 | Battery Charger | 0°C to 70°C (Guaranteed Performance) | The operating range extends from -40°C to 85°C. It has a thermal protection feature that reduces charging current if the internal temperature exceeds 120°C. |
| MIC5219 | Voltage Regulator | -40°C to 125°C | This 3.3V regulator has a wide operating range, suitable for various environmental conditions. |
| ESP32 WROVER-I | MCU | -40°C to 85°C | Most ESP32-WROVER models operate in this range, making them robust for embedded applications. |
| PAM8403 | Audio Amplifier | -40°C to 85°C | Includes over-temperature protection that shuts down the device if the internal temperature reaches 140°C. |
| CH340 | USB to UART | -40°C to 85°C | This is the standard range for most variants of this converter chip. |
| PCF8574 | I/O Expander | -40°C to 85°C | This component for input/output port expansion operates in a standard industrial range. |
| UDA1334A | Audio DAC | -20°C to 85°C | The digital-to-analog audio converter has a slightly more restricted operating range at the lower end compared to other components. |
| TFT Display | Display | Generally, 0°C to 50°C (Typical) | Exact ranges can vary depending on the manufacturer, but this is a common range for consumer TFT displays. Operation outside this range can affect visibility and lifespan. |
| Micro SD Card | Storage | -25°C to 85°C (Typical for Industrial Grade) | Consumer-grade cards may have a more limited range (e.g., 0°C to 70°C). The exact range depends on the specification of the card used. |
| Blue LED | Indicator | -40°C to 85°C (Typical) | Light-emitting diodes (LEDs) typically have a very wide operating range. |

In Terms of Gaming:

It includes 11, YES eleven emulators, like NES, Game Boy, Game Boy Color and More.

Sector Focus:

The project is clearly focused on the maker or DIY sector. The target audience are:

Students and Electronics Hobbyists: People who want to put their hardware and software knowledge into practice.

Retro Gaming Enthusiasts: Nostalgic gamers who would enjoy not only playing but also building their own console.

Programmers and Developers: Those interested in developing software for embedded systems and optimizing code for hardware with limited resources. (For example, PICO-8 is a virtual machine that emulates a video game console. While it is impossible to replicate with ESP32, there are alternatives such as Fake-8, which is the best open-source way to run PICO-8 games on unsupported platforms).

This is not a product for the end consumer looking for an “out-of-the-box” console, but for someone who values the process of creation and customization.

The differential value of ArcadESP is its low cost, the versatile ESP32 and its open-source nature, which invites a community to modify, improve and share their own versions.

References:

Pebri, “GitHub - pebri86/esplay_micro_hardware: Micro version of esplay hardware, ESP32 based gaming console,” GitHub. https://github.com/pebri86/esplay_micro_hardware

“ESPLay Micro V2,” Handheld ESP32 Game Console | Makerfabs. https://www.makerfabs.com/esplay-micro-v2.html

Link to the repository where the files are hosted:

https://github.com/MrSwampert/Portable-Gaming-Console-with-ESP32